Technical Glass Products has included notes in this specification to assist you in tailoring the content to your project.  These comments will appear on your screen in blue.  To hide them and view only the specification, do the following:

Click the “Office” button and then click the word “Options” OR select the “File” and then click “Options”.   Click “Display” at the left of the dialog box.  Remove the check mark from the “Hidden Text” box.  Then click “OK.”  All blue text will then be hidden.

This document is designed for use by a qualified construction specifier. Proper modifications are required to tailor this specification to your project needs.  Optional text is placed between brackets “[ ].”  Blank underlines “__” represent areas you need to fill in with your project specific information. For assistance, call Technical Glass Products at 1-800-426-0279.

This guide specification is a supplier-specific proprietary product specification using the proprietary method of specifying applicable to project specifications and master guide specifications. This Section specifies “Neopariés® LT” crystallized glass ceramic system supplied by Technical Glass Products further described in Part 2 – Products.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

SECTION  07 42 53 – CERAMIC WALL PANELS - NEOPARIÉS® LT ARCHITECTURAL PANELS

PART 1-GENERAL

  - SUMMARY

- Sections includes:

- Prefabricated crystallized glass ceramic Neopariés LT for interior and exterior installation.

  - REFERENCES

A.	See Section 01420. In addition to requirements shown or specified, comply with applicable provisions of the following for design, materials, fabrication and installation of component parts:

- AAMA/NWWDA 101/I.S.2 (AAMA 101).

  - SUBMITTALS

- General: Submit in accordance with Section 01330.

- Product Data: Submit following:

- Manufacturer’s technical literature for each panel product, accessory and other manufactured products required.

- Samples (optional):  Submit 2 each, 90mm x 90mm x 5mm (3-1/2” x 3-1/2” x 3/16”) for approval showing surface consistency and color.

- Shop Drawings: Show sizes, dimension, sections and profiles of panel units, arrangement and provision for jointing, anchoring and support and other installation details.  Indicate location of panels on setting drawings with number designation corresponding to number marked on each unit.  Show large scale details of edge conditions and typical conditions.

- Maintenance Instructions:

  - Instructions shall contain panel supplier’s recommended cleaning materials and application methods, including precautions in use of cleaning materials which may be detrimental to surfaces if improperly applied.

  - Instruct Owner’s personnel in maintenance procedures.  Allow for minimum 1-hour instruction period.

- Warranties:  Submit manufacturer’s warranty.

  - QUALITY ASSURANCE

- Reference specifications and standards:  Comply with recommendations unless otherwise specified in this Section.  In case of conflict, comply with the most stringent.

- The work hereunder shall be performed by a single entity with unit responsibility for architectural panels, design of anchorage and setting systems, field measurements, submittals, field installation and warranty.

- Allowable Tolerances:

- Except for allowance tolerances in panels as specified, make corners of all panels flush and level with corners of adjacent panels.

- For flat surfaces, the maximum deviation from true plumb or level plane, 3mm (1/8”) in 2400mm (8’) as measured under straight edge placed at any location on surface.

  - DELIVERY, STORAGE, AND HANDLING

- Deliver materials to Project Site in undamaged condition.

- Store panels on wood skids or pallets per manufacturer’s instructions.  Place and stand panels vertically on ends to distribute weight evenly and to prevent breakage or cracking of panels.  Protect stored panels from weather with waterproof, non-staining covers or enclosures which will permit air circulation around panels.

- Handle to prevent chipping, breakage, soiling or other damage.  Do not use pinch or wreaking bars without protecting edges of panels with wood or other rigid materials.  Lift with wide-belt slings whenever possible: do not use wire ropes or ropes containing tar or other substances which might cause staining.  If required, use wood rollers and provide cushion at end of wood slides.

- Protect all materials associated with panel system from weather, moisture, and contamination with foreign materials.

  - JOB CONDITIONS

- Installer shall review installation procedures and coordination of other work with the General Contractor, Manufacturer’s Representative, and subcontractors whose work will be affected by the installation of the architectural panels.

- Comply with minimum temperature and ventilation recommendations of manufacturer for setting materials used.

- Protection:  Protect adjoining work and surfaces during installation hereunder.

PART 2 PRODUCTS

  - MANUFACTURERS AND PRODUCTS

- Acceptable Products and Manufacturers:

- Neopariés® LT architectural panels, supplied by Technical Glass Products, 8107 Bracken Place SE, Snoqualmie, Washington 98065, phone (800.426.0279) fax (425.396.8300), email sales@tgpamerica.com, web www.tgpamerica.com.

- Panels:  “Neopariés® LT“, 5mm thick, including curves, shapes, sizes, etc., as shown on the drawings.

- Maximum sizes:  Flat – 1200mm x 2400mm (47-1/4” x 94-1/2”)

- Color:  White.

- Substitutions:  No substitutions permitted.

  - ACCESSORIES

- Installation as required by applicable codes and Architect’s drawings.

- Adhesives are recommended by panel manufacturer.

- EXECUTION

      - GENERAL

- Coordinate supporting structure with Neopariés® LT installation requirements to make installation and support system as efficient and cost effective as possible.

- Coordinate the work of all adjacent trades to allow for optimum site conditions for the installation of all work.

      - PREPARATION

- Examine all supporting substrates to receive parts of the work specified herein.  Verify all dimensions of in-place and subsequent construction.

- Commencing work indicates acceptance of all substrates.

      - INSTALLATION

- Comply with the requirements of the referenced standards.

- Proportion, mix and apply setting materials and install panels in accordance with referenced standards.

- Place panels in accordance with Architect approved Shop Drawings and where not specifically indicated, secure Architect’s approval of proposed layout, cuts, margins, etc., before proceeding with the work.

- Carefully plan layouts: pattern shall be uninterrupted from one wall and/or surface to the next and through doorways or openings.

- Install panels plumb and true with joints uniform in width and accurately aligned.

- Joint width shall be 8mm – 10mm (5/16” x 7/16”) exterior and 6mm (1/4”) interior or as recommended by the manufacturer.  Clean joints free of loose mastic, dirt, and debris before applying sealant.

- Sealant:  Point joints and seal joints and intersection of panels and rigid dissimilar materials with sealant.  Neatly tool sealants to slight concave surface.  Comply with requirements of Section 07 92 00.

      - PROTECTION AND CLEANING

- Remove and replace panel units which are broken, chipped, stained or otherwise damaged.  Where indicated, remove and replace units which do not match adjoining work.

- Clean all panel work not less than 6 days after completion of work.  Do not use wire brushes, acid type cleaning agents or other cleaning compounds with caustic or harsh fillers.

- Provide final protection and maintain conditions, in a manner acceptable to Fabricator and Installer, which ensures all panel work being without damage, discolorations, or deterioration during subsequent construction and until time of substantial completion.

      - WORKMANSHIP

- The finished surfaces shall be free from damage, flaws, blemishes or other defects detrimental to appearance: have panels, joints and reveals in required pattern, position and alignment and be uniform in plan, texture and finish.

END OF SECTION